Практическое занятие № 4 «Монтаж внутренних сантехнических устройств, кондиционирования воздуха и вентиляции»

1.Read and translate the text

Drainage, Waste, and Vent Installation: Venting

Proper venting of every fixture is an essential part of any DWV system. There are several different kinds of vents. Plumbers must be familiar with each type when installing a DWV system.

First, an individual vent serves a single fixture trap and connects to other vents or open air. Two of these vents can connect together to form a branch vent, which is a vent for a horizontal branch. Individual drains are connected to a horizontal branch by a fixture branch.

Next, large bathroom areas in commercial buildings may use circuit vents, which can serve up to eight fixture traps. Relief vents must be installed with a circuit vent to circulate air between the drainage and venting system. Last, a loop vent is a special circuit vent located on the top floor of a building.

Two special kinds of venting systems usually require approval before installation. First, wet venting uses a single vent for more than one fixture. These are often used in household bathrooms. Second, island venting can be used for fixtures that are not next to a wall, such as some kitchen sinks.

Before a DWV system can be installed, piping must be properly sized. Sizing calculations use drainage fixture units, which measure the number of liters of water per second that flow from a fixture into a drainage system.

2.Answer the questions:

a) What are some segments of a drainage, waste, and vent system inside a building?

b) What are some different types of vents?

c) What grammatical phenomena can you analyze in the text?

d) Find in the text the sentences in the Present Simple. Read them and analyze the verbs.

e) Find in the text the sentences with the modal verbs. Read them.

3.Mark the following statements as true (T) or false (F) having read the textbook passage “Drainage, Waste, and Vent Installation: Venting”

1 – Two individual vents connect in a branch vent.

2 – Circuit vents do not require relief vents.

- – Island venting is usually found in bathrooms.

4.Match the words (1-6) with the definitions (A-F)

| 1 – individual vent | 4 – loop vent |
| --- | --- |
| 2 – horizontal branch | 5 – branch vent |
| 3 – relief vent | 6 – circuit vent |

A a vent that ends by connecting to a stack vent instead of a vent stack

B a vent that connects to between two and eight fixture traps

C a pipe that connects two or more fixture drains to the main portion of a system

D a vent that connects a fixture trap to another vent or ends in open air

E a vent that connects one or more individual vents with a stack

F a pipe that circulates air between a drainage and a vent system

5.Fill in the blanks with the correct words and phrases from the word bank

Word bank: venting, island venting, drainage fixture unit, fixture branch

1 You will need to use _____________ to reach the sink in the middle of the room.

2 Be sure to know the correct ___________ measurement for that fixture.

3 You can use a single ____________ system for all the fixtures in the room.

4 You need to install a(n) ____________ to connect those drains to the system.

6.Translate the following words and phrases

individual vent, loop vent, horizontal branch, branch vent, relief vent, circuit vent, venting, island venting, drainage fixture unit, fixture branch

7.Make up a dialogue on the theme “Drainage, Waste, and Vent Installation: Venting”.

8.Retell the text (10 -15 sentences).